TOYOTA MOTOR MANUFACTURING

NORTH AMERICA

Supplier	Packaging

Policies	&	Standards

September 2002

Table of Contents

1.

Background...........................................................................  2

2.

Purpose..................................................................................2

3.

Scope... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... .... ... ... ... ...2

4.

Packaging Operations Manual.....................................................2

5.

Policy... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... .....3

Expectations........................................................................... .4

6.

7.

Process (Excludes NUMMI & TMMC)... ... ... ... ... ... ... ... ... ... ... ... ... ... ...5

8.

Standards.................................................................................6

Appendix A

Tote Standards... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ....7

Appendix B

Label Standards... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ... ...9

Appendix C - Rack Standards... ... ... ... ... ... ... ..... ... ... ... ... ... ... ... ... ...12

9/2002

Toyola Molor Manufacluring  Norlh America

Packaging Policies & 5landards

1

1.

Background

Toyota generally requires that all parts shipments to our manufacturing facilities be made

using returnable packaging.  This packaging includes steel racks, plastic totes and dunnage that may be required to secure and protect the parts within the tote.

Recently, TMMNA has reflected on the total cost of returnable packaging on past vehicle

programs.  The reflection highlighted 2 main points:

•

•

There are significant cost reduction opportunities available.

A major portion of the total cost is determined by the procurement activities of our parts suppliers.

As a result, returnable packaging has been identified as a major cost reduction

opportunity for you, the supplier, to meet Toyota expectations for total cost reduction. This manual has been developed to help you meet those expectations.

2.

Purpose

This manual aims to:

•

•

•

•

Communicate TMMNA Returnable Packaging Policy

Communicate TMMNA Procurement Expectations

Communicate TMMNA Standards

Provide a reference

3.

Scope

4.

Packaging  Operations Manual

This document is for Policies and Standards.  Each NAMC has its own Pkg Operations

Manual to cover Packaging Quality and Operational issues in addition to this document. If you find anything conflicting between this document and the NAMC Pkg Operations Manual’s, please contact TMMNA PC for clarification.

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

2

Policy & Expectations

All NAMC’s except NUMMI

Payment Process

TMMAL, TMMBC, TMMK, TMMI, & TMMWV

Specifications

Racks

All NAMC’s

Labels

All NAMC’s

Totes

All NAMC’s (NUMMI has additional types &

sizes

5.

Policy

TMMNA’s policy regarding returnable packaging is outlined in our Terms and Conditions.

It has been copied verbatim for your reference.

•

Reference Terms and conditions, Section 2.4d

2.4 (d) Packing Requirements

1.

Unless otherwise instructed by Toyota Party, all items are to be prepared

and packed according to packaging specifications provided by Toyota Party, and are to meet all carriers requirements. The quantity of packaging is determined by Toyota Party, and any additional quantities are Supplier’s responsibility.

In the event returnable packaging is deemed by Toyota Party to be advisable or necessary for the delivery of items (“Returnable Packaging”), TMMNA shall reimburse Supplier for the acquisition costs of such Returnable Packaging on mutually acceptable terms. The Returnable Packaging shall be the property of Supplier unless otherwise designated by TMMNA. The price for such Returnable Packaging shall be set forth in a separate Purchase Order and paid for in accordance with the general payment terms as set forth in Section 2.3. Supplier  is responsible for

2.

cleaning,  replacing or repairing any lost or damaged Returnable

Packaging.

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

3

6.

Expectations

Each and every supplier is expected to:

1.  Take full responsibility of their returnable packaging cost and to commit to reducing it significantly.

2.  Select the lowest cost option available that will ensure part quality.

Specifically:

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

4

Cost

Control

1.  Reuse obsolete or extra packaging rather than purchase new.

2.  Competitively bid at least 3 sources for all packaging unless the source has been pre-determined by Toyota.

3.  Consider the total cost of the packaging when making your sourcing decision including freight, labels, kanban holders, etc.

Simple

Design

4.  Utilize standard tote & rack sizes unless part design or other criteria make in impractical.

5.  If a non-standard size packaging is approved by Toyota, follow all other standard specifications such as color, stacking feature, etc.

6.  Design / redesign packaging to reduce cost. Some items to consider are:

a.  Eliminating or reducing dunnage

b.  Using alternate, less costly materials c.   Increasing box efficiency.

d.  Simplifying the design for ease of manufacture e.  Utilizing same design for each NAMC

1.  Target less than 2 hours production per container. (Note: some NAMC’s may have lower targets.)

2.  Do not exceed a total weight of 15 kg. Per tote including dunnage & parts. (Note: Some NAMC’s may have a maximum box weight limit that is less than 15 kg.)

3.  During the design phase consider the following options in the sequence shown below.

a.  Option 1: Utilize the most efficient, smallest standard tote first without dunnage.

b.  Option 2: Utilize the most efficient, smallest standard tote with dunnage

c.   Option 3: Utilize the most efficient, smallest standard rack without dunnage. Target 48 x 45 footprint.

d.  Option 4: Utilize the most efficient, smallest standard rack with dunnage. Target 48 x 45 footprint.

Procurement

Process

1.  Actively seek potential packaging suppliers in order to find the most competitive sources, other than TMMNA sourced packaging (totes)

2.  Begin the procurement process (up to the point of ordering) as soon as the packaging has been approved so that the best price can be achieved.

3.  Do not place the order until authorization has been received from the

NAMC.

4.  When a quote is requested by TMMNA, utilize the WARP Returnable

Packaging quote form and provide as much detail as possible.

5.  Consider lead time when quoting to ensure on time delivery.

7.

Process

Responsible

Excludes: NUMMI & TMMC

Note: N = Month of SOP

Target Timing

General Flow

NAMC PC

N - 14

START

NAMC PC

N - 14

SUPPLIER

N - 10

NAMC PC

N-  9

Review Proposal

Approve

NAMC PC

N - 9

SUPPLIER

N - 8

NAMC PC

N - 7

Review Pkg Trial

Approve

SUPPLIER

N - 6

SUPPLIER

N - 6

Review Pkg Proposal / Authorize Packaging

NAMC PC

N - 5

Approve

SUPPLIER

N - 5

SUPPLIER

N - 2

SUPPLIER

N - 2

RFQ from TMMNA

No

Received

Yes

SUPPLIER

N - 2

NA PURCHASING

N + 2

Approved by TMMNA Purchasing

Yes

NA PURCHASING

N +2

PO Received from

Yes

SUPPLIER

N + 3

SUPPLIER

N + 4

END

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

5

Receive Payment from TMMNA

Invoice TMMNA

TMMNA 	No

Contact TMMNA Buyer

No

Negotiate with

TMMNA Buyer

Send Proof of Delivery & Quote to

TMMNA Purchasing

Contact TMMNA Buyer

Receive Packaging

Order Packaging

Reject

Contact NAMC PC Pkg Specialist

Submit Final Pkg Proposal

Receive Quotes from Packaging

Suppliers

Reject

Contact NAMC PC Pkg Specialist

Prepare / Ship Pkg Trial

Authorize Packaging  Trial

Reject

Contact NAMC PC Pkg Specialist

Supplier Submit Proposal to

NAMC

Mail Preliminary  Proposals  to

Supplier

8.

Standards

In an effort to reduce overall logistics costs, Toyota has implemented standard totes,

labeling, and rack specifications.

The standard totes have been in place since 1997.  The biggest recent change is the

standardization of the color between NAMC’s to Charcoal Grey. It does not obsolete any containers of different colors.  It is a running change for future orders.  This improves efficiency of container manufacturers to reduce lead time and cost.

The container identification standards are important to help improve the visual controls of

the container, to improve the return accuracy to our Suppliers.

for the re-use of containers in the future.

It also improves flexibility

The biggest change to our standard containers is the agreement of rack standards

between all NAMC’s.  The 2 critical points of agreement include the dimensions of the racks, and the stacking points.  The reason for standard racks is to reduce overall logistics cost by doing the following:

a.

Improving stack ability of the trailers to increase the cubic efficiency of our

routes

Allows for competitive bidding

Facilitates the re-use of racks for future models

b.

c.

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

6

Appendix A:  Tote, Pallet, & Hot Stamp Standards

Tote Standards:

* = Cut n weld container, NUMMI does not utilize these sizes

•

•

•

•

Container Color: Charcoal Grey (NUMMI Blue)

Labels / Tote: See Above. NUMMI: 2

Kanban Holders / Tote: 2

Hot Stamps:

o	TMMAL, TMMBC, TMMI, TMMK, TMMWV: “TOYOTA SHIPMENT”

o	TMMC: “TMMC [TOYOTA]” 2/tote

1/tote

o	NUMMI: NUMMI Logo, Property of New United Motor Manufacturing, Container Height 2/tote

NUMMI utilizes additional sizes. Please refer to following page for NUMMI only containers.

Pallet & Top Cap Standards

•

•

Color: Black

Hot Stamps: 2. One on each 45” side of pallets & top caps

Hot stamp information is the same as above.  The hot stamp is surrounded by a white square

that simulates a load stripe.

NUMMI has hot stamp “Property of NUMMI” on 48” side.

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

7

#

Dimension:  In.

Dimension: mm

Orbis

Schaefer

L

W

H

L

W

H

Plt

Toyota

48.75

45.75

6.8

1238

1162

165

45 x 48-B MI CISF LP (with lip)

AFP 4845 (with lip)

Plt

NUMMI

48.75

45.75

6.8

1238

1162

165

45 x 48-C CISF (with lip)

N/A

Top

48.75

45.75

1.5

1238

1162

50

45 x 48 MP-2 Top Cap

CISC

AFT4845

#

Dimension:  In.

Dimension: mm

Labels / Tote

Orbis

Schaefer

L

W

H

L

W

H

1

12

15

4

305

381

102

2

NSO 1215-4RQ

AF 1215-04XC

2

12

15

7.5

305

381

191

4

NSO 1215-7RQ

AF 1215-07XC

3

24

15

4

610

381

102

2

NSO 2415-4RQ

AF 2415-04XC

4

24

15

7.5

610

381

191

4

NSO 2415-7RQ

AF 2415-07C

5

24

15

11

610

381

279

4

NSO 2415-11RQ

AF 2415-11XC

6

24

15

14.5

610

381

368

4

SO 2415-14RQ

AF 2415-14C

*7

36

15

4

914

381

102

2

NSOT 3615-4RQ

AF 3615-04CWC

*8

36

15

7.5

914

381

191

4

NSOT 3615-7RQ

AF 3615-07CWC

*9

36

15

11

914

381

279

4

NSOT 3615-11RQ

AF 3615-11CWC

*10

48

15

4

1219

381

102

2

NSOT 4815-4RQ

AF 4815-04CWC

11

48

15

7.5

1219

381

191

4

SO4815-7RQ

AF 4815-07XC

12

48

15

11

1219

381

279

4

SO4815-11RQ

AF 4815-11XC

13

24

22.5

7.5

610

572

191

4

NSO 2422-7RQ

AF 2422-07XC

14

24

22.5

11

610

572

279

4

NSO 2422-11RQ

AF 2422-11C

15

24

22.5

14.5

610

572

368

4

NSO2422-14RQ

AF 2422-14C

NUMMI TOTE STANDARDS

NUMMI utilizes TMMNA Standard containers for local California suppliers.

NUMMI for most suppliers are listed below.

The primary containers utilized by

Orbis FC-4845 Folding Containers

Orbis FC-4845 Folding Containers with Storage Bottom (containers with storage bottoms for custom interiors)

Buckhorn Bi-Color Stack & Nest Containers

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

8

Outside Dimensions

Inside Dimensions

Model

Length

Width

Height

Length

Width

Storage

Bottom Height

Weight Pounds (Kg)

K.D. Ratio

BC24229

24.0 (610)

22.5 (572)

9.0 (229)

21.8

20.5

4.5

7.1 (3.2)

2:1

BC242211

24.0 (610)

22.5 (572)

11.0 (279)

21.8

20.5

5.5

7.6 (3.4)

2:1

Outside Dimensions

Inside

Dimensions

Product Clearance

Model

Length

Width

Height

Stack

Height

Length

Width

Assembled

Collapsed

Collapsed

Height

Weight Pounds (Kg)

K.D. Ratio

FC2415-111-4

24.0 (610)

15.0 (381)

11.1 (282)

10.8

22.5

13.6

10.4

3.6

6.2

7.0 (3.2)

1.8:1

FC2415-147-4

24.0 (610)

15.0 (381)

14.7 (373)

14.4

22.5

13.6

14.0

3.6

6.2

8.5 (3.9)

2.4:1

FC2415-147-7

24.0 (610)

15.0 (381)

14.7 (373)

14.4

22.5

13.6

14.0

7.0

9.8

8.4 (3.8)

1.5:1

Outside Dimensions

Inside

Dimensions

Model

Length

Width

Height

Stack

Height

Length

Width

Product

Clearance

Collapsed

Height

Weight Pounds (Kg)

K.D. Ratio

FCSO1215-4

12.0

(305)

15.0

(381)

4.0

(102)

3.6

13.0

9.4

3.0

NA

2.0

(.91)

1:1

FC1215-075

12.0

(305)

15.0

(381)

7.5

(191)

7.2

13.5

10.6

6.8

2.6

3.7

(1.7)

3.1:1

FC1215-089

12.0

(305)

15.0

(381)

8.9

(226)

8.6

13.5

10.6

8.2

3.5

4.0

(1.8)

2.7:1

FC2415-075

24.0

(610)

15.0

(381)

7.5

(191)

7.2

22.5

13.6

6.8

2.6

5.3

(2.4)

3.1:1

FC2415-089

24.0

(610)

15.0

(381)

8.9

(226)

8.6

22.5

13.6

8.2

2.6

6.0

(2.7)

3.7:1

FC2415-111

24.0

(610)

15.0

(381)

11.1

(282)

10.8

22.5

13.6

10.4

2.6

6.7

(3.0)

4.7:1

FC2415-147

24.0

(610)

15.0

(381)

14.7

(373)

14.4

22.5

13.6

14.0

3.5

8.3

(3.8)

4.5:1

Appendix B:  Container  Identification

3)

KD Totes (NUMMI):

2 sides

(NUMMI NETWORK)

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

9

Item

Container ID Label Specifications

Dimension

6.25" x 1"

Location

1)  Stack Only Totes:  4 sides, except 12" x 15" x 4" - 	2 sides sides

2)  Nesting Totes:  4 sides

Colors

#

Label Color

Font

Color

Cross Dock

(NA Netw ork)

Consolidation Center

1

Blue

Process Blue

White

GCD

Detroit

2

Red

Red 187

White

RCD

Southern California

3

Green

Green 354

White

BCD

Northern California

4

Orange

Orange 144

Black

KCD

El Paso

5

Purple

Pantone Purple

White

OCD

6

Yellow

Pantone Yellow

Black

MCD

MCD

7

Pink

Pantone 223C

Black

Direct

8

White

Black

Chicago

Direct & Cross dock label:

Some suppliers ship to 1 NAMC through both a cross dock and direct.  In this case, the top half of the label is the color of the cross dock, the bottom half of the label is Pink 223c to reflect direct.  Font color is always black.

BLACK 	FONT

Layout

See pages 2 & 3 for each NAMC's label layout.

6.25"

1"

Container  Identification Continued

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

10

NAMC

LABEL  LAYOUT

TMMI

All Fonts:  56 point Arial Bold;  NAMC Code, Supplier Code;  TLMS Code

TMMK

All Fonts:  56 point Arial Bold;  NAMC Code, Supplier Code, TLMS Code

TMMWV

NAMC Code:  36 point font;  Supplier Code: 56 point font;  TLMS Code:  56 point font; All Arial Bold

TMMC

GCD

NAMC Code:  36 point font;  Supplier Code: 56 point font;  TLMS Code:  56 point font; Cross dock:  20 point font;  All Arial Bold

6.25"

1"

C 	0000A 	SUPLA

6.25"

1"

WV 00000	SUPLA

6.25"

1"

K 	00000	SUPLA

6.25"

1"

I	00000	SUPLA

Container  Identification Continued

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

11

NAMC

LABEL  LAYOUT

TMMAL

NAMC Code:  36 point font;  Supplier Code: 56 point font;  TLMS Code:  56 point font; All Arial Bold

NUMMI

0000-0	CITY,	ST

TLMS CODE:  49 pt font;  Supplier Code:  19 point font;  Supplier Name 29 point font; City/State:  29 point font;  All Arial Bold

TMMBC

NAMC Code:  36 point font;  Supplier Code: 56 point font;  TLMS Code:  56 point font; All Arial Bold

6.25"

1"

BC 00000	SUPLA

6.25"

1"

SUPLA 	SUPPLIER NAME

6.25"

1"

AL 	00000	SUPLA

Appendix C:

Rack Standards

tall.  It takes into consideration 1.75" of nesting

recessed bottom.

Paint Type:  ESD

Stacking Cap location:

Top, Recessed Bottom (1", 25 mm)

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

12

Item

Detail

Footprint

The 13 footprints shaded box's & denoted with "0" are standard sizes. Always specify 45" x 48" footprint as first choice.

(4572mm)

Height

The height is based on a trailer opening of 98" for the stacking cap, and 1" of nesting for the

Color

All NAMC's except NUMMI:  Light Grey

NUMMI: 	Blue

Rack

ID

ID Color:   Black,  NUMMI Color:  White

Size:  1.5"  (38mm) Block Capitals Font

Information: TOYOTA or NUMMI, NAMC Code,  Supplier Code, TLMS Code

ID Location:	All 4 sides of the base.

Sequential Number: 	All racks must be sequentially numbered.  Two digit year code preceding the number.  Located on the corner post.

Stacking

Caps

Pin Post Design:  Livonia Tool, 1738 B

Rack

Frame

Frame:  2" x 2" 11 gauge Steel

Stack

Height

Height

(in)

Height

(mm)

1

98.0

2489

2

50.0

1270

3

34.0

864

4

26.5

673

5

21.5

546

6

18.5

470

Standard Rack

Footprints

Length

30''

(762mm)

45"

(1143m m )

60"

(1524m m )

75"

(1905mm)

90"

(2286m m )

105"

(2667m m )

120"

(3048m m )

150"

(3810m m )

180"

W

i d t h

32"

(813m m )

0

48"

(1219mm)

0

0

0

0

0

0

0

0

0

96"

(2438mm)

0

0

0

Note :  45 x 48 footprint should  alw ays be first choice

Rack Standards  continued

Toyota Motor Manufacturing North America

Packaging Policies & Standards

9/2002

13

Item

Detail

Kanban

Holders

Kanban holders on 2 adjacent sides located near corner post.  Holders must be visible.

Contact specific NAMC to confirm location and specification for kanban holder.

Bottom

Strap

4 way entry

Opening Clearance:  3.5"  (89mm)

1" x 2" (25mm x 51mm) bottom strap.  Located around the perimeter of the base.

Walk In

Feature

Walk in design should target  an 18" width and a depth of 1/2 the distance into the rack.

Knock Down Design

Current Standard:  Pin on the bottom.  Working to develop a design with the pin on the top

CDnmdp|etEWE|ds  A O U 	11	items

"

"

11

Item  5

I    ZEJD="'   ""

11   ""o.IOTγ-=

fγ

Balla  d  	m     1OF1

9/2002

Toyola Molor Manufacluring  North America

Packaging Policies & 5landards

1

"

_Ite2

HEIGHTS  (OD  01  R ck	H Ight)

/	elght	" c/

"U

'" 	NTEs:G |nd	q||shq nPd CO ne S Grind  welds  flush  in  stcking    S

h	Item  1

(7 Item   2

\ltemv

0"   ees)

12  I   1

Po;nl

c>liDht  G

11

4

2" 2" 11       00

Holght m;nus 2.9"

10

4

2" 2" 11       00

"

9

8

2

2

2" 2" 11       00

2"   x  2" 11  00

18" Leno\h

41"  Leng\h

44"  Lenath

7

4

2" x  2" 11  00

6

ι

'" 2" 11       00

41"  nD\h

5

.z. "	ga

4

8

1/+" Pla\e

1" 2

3

8

"κ2" 11   go

JS L.nalh

2

6

3/16"  An

z' z' 3.5"	3/16" /ln ale

I   4   I   Pucch 	I     Uv  0'    '0 I  173       Stack;na   Ca

ILL     OF  MATERIALS

FmURN1lAMLEEE/NHA1BSBDNNEENEHTEEEExAEAmKxERRXlxMElE S[AMNE1llSL lPMlANEENEEElHFLElE1DE]     45 ..22          x  48"a a        R ck    ase

T          N"'"  ...	'"

25  Atlant;c   Avenue

EP' !   ' MaEl B     EE     Cc'nge"      KY    41 B

".	'""1" /l/2|lmT  MNA.PK-R 	1-

Item    11

r'tem

/

/

3Õ

1ri

Stck      Height

Height (in)

Height (mm)

1

98

2489

50    "

127

3

34

864

4

25

673

5

21 5

546

6

18.5

470

95'